| Index | Hermes name | Hermes FPGA pin | HF2 Name | TX2 Name | CVA9 J2 pin | CVA9 name | CVA9 FPGA pin | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | _122MHz | 150 | 122_88MHZ | - | 5 | EG_P1 | W16 | 122.88 MHz input from VCXO |
| 2 | OSC_10MHZ | 89 | EXT_OSC_10MHZ | - | 6 | EG_P35 | N9 | 10MHz reference input clock |
| 3 | FPGA_PLL | 144 | VCXO_CTL | - | 65 | EG_P28 | U22 | VCXO control voltage PWM output |
| 4 | ATTN_DATA | 39 | SPI_DATA | SPI_DATA | 62* | EG_P58 | V20 | data output to attenuator |
| 5 | ATTN_CLK | 22 | SPI_CLK | SPI_CLK | 64* | EG_P59 | U20 | clock output to attenuator |
| 6 | ATTN_LE | 69 | ATTN_LE | - | 66 | EG_P60 | P18 | latch enable output to attenuator |
| 7 | INA0 | 185 | INA0 | - | 9 | EG_P3 | AA13 | input data from ADC |
| 8 | INA1 | 184 | INA1 | - | 11 | EG_P4 | AA14 | input data from ADC |
| 9 | INA2 | 183 | INA2 | - | 13 | EG_P5 | Y14 | input data from ADC |
| 10 | INA3 | 177 | INA3 | - | 15 | EG_P6 | AB15 | input data from ADC |
| 11 | INA4 | 176 | INA4 | - | 17 | EG_P7 | AA15 | input data from ADC |
| 12 | INA5 | 173 | INA5 | - | 19 | EG_P8 | Y15 | input data from ADC |
| 13 | INA6 | 171 | INA6 | - | 23 | EG_P9 | Y16 | input data from ADC |
| 14 | INA7 | 169 | INA7 | - | 25 | EG_P10 | AB17 | input data from ADC |
| 15 | INA8 | 166 | INA8 | - | 27 | EG_P11 | AA17 | input data from ADC |
| 16 | INA9 | 164 | INA9 | - | 29 | EG_P12 | Y17 | input data from ADC |
| 17 | INA10 | 162 | INA10 | - | 31 | EG_P13 | AB18 | input data from ADC |
| 18 | INA11 | 161 | INA11 | - | 35 | EG_P14 | AA18 | input data from ADC |
| 19 | INA12 | 160 | INA12 | - | 37 | EG_P15 | AA19 | input data from ADC |
| 20 | INA13 | 159 | INA13 | - | 39 | EG_P16 | Y19 | input data from ADC |
| 21 | INA14 |  | INA14 | - | 41 | EG_P17 | AB20 | input data from ADC |
| 22 | INA15 |  | INA15 | - | 43 | EG_P18 | Y20 | input data from ADC |
| 23 | LTC2208_122MHz | 152 | ADC_CLKA | - | 7 | EG_P2 | V15 | buffered clock from ADC |
| 24 | OVERFLOW | 146 | OVFLA | - | 49 | EG_P21 | AB21 | overflow input from ADC |
| 25 | RAND | 145 | RAND | - | 47 | EG_P20 | AB22 | random enable to ADC |
| 26 | PGA | 143 | PGA | - | 51 | EG_P22 | AA22 | PGA enable to ADC |
| 27 | DITH | 189 | DITHER | - | 56 | EG_P55 | V18 | dither enable to ADC |
| 28 | SHDN | 194 | - | - | - | - | - | shutdown output to ADC |
| 29 | DAC_ALC | 137 | - | DAC_ALC | 16 | EG_P39 | N6 | DAC PWN output level |
| 30 | DACD0 | 195 | - | DACD0 | 50 | EG_P53 | U16 | output data to DAC |
| 31 | DACD1 | 196 | - | DACD1 | 48 | EG_P52 | U15 | output data to DAC |
| 32 | DACD2 | 197 | - | DACD2 | 46 | EG_P51 | V16 | output data to DAC |
| 33 | DACD3 | 200 | - | DACD3 | 42 | EG_P50 | V14 | output data to DAC |
| 34 | DACD4 | 201 | - | DACD4 | 40 | EG_P49 | T14 | output data to DAC |
| 35 | DACD5 | 202 | - | DACD5 | 38 | EG_P48 | T13 | output data to DAC |
| 36 | DACD6 | 203 | - | DACD6 | 36 | EG_P47 | T12 | output data to DAC |
| 37 | DACD7 | 207 | - | DACD7 | 34 | EG_P46 | R14 | output data to DAC |
| 38 | DACD8 | 214 | - | DACD8 | 30 | EG_P45 | P14 | output data to DAC |
| 39 | DACD9 | 216 | - | DACD9 | 28 | EG_P44 | T8 | output data to DAC |
| 40 | DACD10 | 217 | - | DACD10 | 26 | EG_P43 | T7 | output data to DAC |
| 41 | DACD11 | 218 | - | DACD11 | 24 | EG_P42 | R6 | output data to DAC |
| 42 | DACD12 | 219 | - | DACD12 | 20 | EG_P41 | R5 | output data to DAC |
| 43 | DACD13 | 221 | - | DACD13 | 18 | EG_P40 | P6 | output data to DAC |
| 44 | CBCLK | 240 | CBCLK | CBCLK | 59 | EG_P25 | W22 | output to CODEC |
| 45 | CLRCIN | 236 | CLRCIN | CLRCIN | 61 | EG_P26 | W21 | output to CODEC |
| 46 | CLRCOUT | 232 | CLRCOUT | CLRCOUT | 63 | EG_P27 | V21 | output to CODEC |
| 47 | CDIN | 239 | CDIN | CDIN | 55 | EG_P23 | Y22 | output to CODEC |
| 48 | CMCLK | 223 | CMCLK | CMCLK | 52 | EG_P54 | U17 | master clock to TLV320 audio CODEC |
| 49 | CMODE | 230 | - | - | - | - | - | mode select output to CODEC (I2C or SPI) |
| 50 | nCS | 231 | PH_CODEC_nCS | - | 60 | EG_P57 | V19 | chip select output to CODEC |
| 51 | nCS | 231 | - | MIC_CODEC_nCS | 57 | EG_P24 | Y21 |  |
| 52 | MOSI | 226 | SPI_DATA | SPI_DATA | 62* | EG_P58 | V20 | SPI data output to CODEC |
| 53 | SSCK | 224 | SPI_CLK | SPI_CLK | 64* | EG_P59 | U20 | SPI clock output to CODEC |
| 54 | CDOUT | 235 | - | MIC_CDOUT | 45 | EG_P19 | AA20 | mic data input from CODEC |
| 55 | PHY_TX0 | 50 | - | - | - | ENET_TXD0 | M21 | transmit data output to PHY |
| 56 | PHY_TX1 | 49 | - | - | - | ENET_TXD1 | M20 | transmit data output to PHY |
| 57 | PHY_TX2 | 45 | - | - | - | ENET_TXD2 | N21 | transmit data output to PHY |
| 58 | PHY_TX3 | 44 | - | - | - | ENET_TXD3 | N20 | transmit data output to PHY |
| 59 | PHY_TX_EN | 41 | - | - | - | ENET_TXEN | K22 | transmit enable output to PHY |
| 60 | PHY_TX_CLOCK | 43 | - | - | - | ENET_GTX_CLK | L22 | transmit clock output to PHY |
| 61 | PHY_RX0 | 6 | - | - | - | ENET_RXD0 | AB6 | receive data input from PHY |
| 62 | PHY_RX1 | 13 | - | - | - | ENET_RXD1 | AB5 | receive data input from PHY |
| 63 | PHY_RX2 | 18 | - | - | - | ENET_RXD2 | AA7 | receive data input from PHY |
| 64 | PHY_RX3 | 21 | - | - | - | ENET_RXD3 | AB7 | receive data input from PHY |
| 65 | RX_DV | 113 | - | - | - | ENET_RX_DV | V9 | receive data valid input from PHY |
| 66 | PHY_RX_CLOCK | 31 | - | - | - | ENET_RX_CLK | M8 | receive data clock input from PHY |
| 67 | PHY_CLK125 | 149 | - | - | - | - |  | 125MHz clock input from PHY PLL |
| 68 | PHY_INT_N | 55 | - | - | - | ENET_INTn | N8 | interrupt input from PHY |
| 69 | PHY_RESET_N | 56 | - | - | - | ENET_RSTn | K21 | reset output to PHY |
| 70 | CLK_25MHZ | 33 | - | - | - | - | - | 25MHz clock input from PHY oscillator |
| 71 | PHY_MDIO | 52 | - | - | - | ENET_MDIO | AB12 | MDIO data to/from PHY |
| 72 | PHY_MDC | 52 | - | - | - | ENET_MDC | V13 | MDIO clock to PHY |
| 73 | SCK | 68 | - | - | - | - | - | clock output to MAC eeprom |
| 74 | SI | 38 | - | - | - | - | - | data output to MAC eeprom SI pin |
| 75 | SO | 70 | - | - | - | - | - | data input from MAC eeprom SO pin |
| 76 | CS | 87 | - | - | - | - | - | chip select output to MAC eeprom |
| 77 | NCONFIG | 63 | - | - | - | RECONF | G6 | reload FPGA from config prom when high |
| 78 | ADCMOSI | 78 | - | - | - | - | - | data output to serial ADC |
| 89 | ADCCLK | 82 | - | - | - | - | - | clock output to serial ADC |
| 80 | ADCMISO | 57 | - | - | - | - | - | data input from serial ADC |
| 81 | nADCCS | 118 | - | - | - | - | - | chip select output to serial ADC |
| 82 | SPI_SDO | 81 | - | - | - | - | - | SPI data output to Alex/Apollo |
| 83 | SPI_SCK | 83 | - | - | - | - | - | SPI clock output to Alex/Apollo |
| 84 | J15_5 | 114 | - | - | - | - | - | SPI RX data stobe to Alex or Apollo enable |
| 85 | J15_6 | 117 | - | - | - | - | - | SPI TX data strobe to Alex or Apollo reset_n |
| 86 | PTT | 98 | - | FPGA_PTT_N | 8 | EG_P36 | P9 | active-low PTT input |
| 87 | KEY_DOT | 76 | - | FPGA_DOT_N | 78 | EXP_PRESENT | P19 | active-low dot input from J6 |
| 88 | KEY_DASH | 73 | - | FPGA_DASH_N | 58 | EG_P56 | W19 | active-low dash input from J6 |
| 89 | FPGA_PTT | 84 | - | PA_KEY | 14 | EG_P38 | M6 | active-high PTT output to J11 and J16 |
| 90 | MODE2 | 110 | - | - | - | DIP_SW4 | E16 | input: jumper is on Hermes J13 when low |
| 91 | ANT_TUNE | 94 | - | - | - | - | - | input from ATU |
| 92 | IO1 | 95 | - | - | - | - | - | active-high output to mute audio amp |
| 93 | IO2 | 46 | - | - | - | - | - | PTT input from Apollo |
| 94 | IO4 | 88 | - | - | - | - | - | aux digital input from J16 |
| 95 | IO5 | 99 | - | - | - | - | - | aux digital input from J16 |
| 96 | IO6 | 100 | - | - | - | - | - | aux digital input from J16 |
| 97 | IO8 | 126 | - | - | - | - | - | aux digital input from J16 |
| 98 | USEROUT0 | 135 | - | - | - | - | - | aux open-collector digital outputs to J16 |
| 99 | USEROUT1 | 134 | - | - | - | - | - | aux open-collector digital outputs to J16 |
| 100 | USEROUT2 | 133 | - | - | - | - | - | aux open-collector digital outputs to J16 |
| 101 | USEROUT3 | 132 | - | - | - | - | - | aux open-collector digital outputs to J16 |
| 102 | USEROUT4 | 131 | - | - | - | - | - | aux open-collector digital outputs to J16 |
| 103 | USEROUT5 | 128 | - | - | - | - | - | aux open-collector digital outputs to J16 |
| 104 | USEROUT6 | 127 | - | - | - | - | - | aux open-collector digital outputs to J16 |
| 105 | Status_LED | 80 | - | - | - | USER_LED8 | D21 | active-high configured LED output |
| 106 | DEBUG_LED1 | 9 | - | - | - | USER_LED1 | B17 | active-low user/debug LED output |
| 107 | DEBUG_LED2 | 93 | - | - | - | USER_LED2 | E19 | active-low user/debug LED output |
| 108 | DEBUG_LED3 | 142 | - | - | - | USER_LED3 | E21 | active-low user/debug LED output |
| 109 | DEBUG_LED4 | 139 | - | - | - | USER_LED4 | B21 | active-low user/debug LED output |
| 110 | DEBUG_LED5 | 188 | - | - | - | USER_LED5 | C20 | active-low user/debug LED output |
| 111 | DEBUG_LED6 | 37 | - | - | - | USER_LED6 | C21 | active-low user/debug LED output |
| 112 | DEBUG_LED7 | 111 | - | - | - | USER_LED7 | D19 | active-low user/debug LED output |
| 113 | DEBUG_LED8 | 112 | - | - | - | - | - | active-low user/debug LED output |
| 114 | DEBUG_LED9 | 103 | - | - | - | - | - | active-low user/debug LED output |
| 115 | DEBUG_LED10 | 106 | - | - | - | - | - | active-low user/debug LED output |
| 116 | - | - | DRV_CLK_OUT_N | - | 67 | EG_P29 | U21 | output to HF2: drive 122.88MHZ to J1 pin 5 |
| 117 | - | - | - | DAC_CLK | 3 | RESET_EXPn | U13 | output to TX2: clock to DAC |
| 118 | - | - | - | EN_RX_ANT | 12 | EG_P37 | M7 | output to TX2: t/r switch |
| 119 | - | - | - | - | - | DDR3_CLK_50MHZ | H13 | input from 50MHZ oscillator |
| 120 | - | - | - | - | - | CLK_24MHZ | M9 | input from 24MHZ oscillator |