Increase you Resistor Inventory with this Resistor Search Program

MatLab Code and Resistor Lists

I generated and run the code in MatLab, but it can be adapted to C or other languages.

% ******************************Start Code*********************************

% *                                    *

% * Title:  ReSearch.m                         *

% * Abstract: A program to calculate selected parallel resistances from a *

% *      list of resistors on hand and calculate a desired      *

% *      resistance by selecting resistance values from the new   *

% *      resistance list.                      *

% *                                    *

% *                                    *

% * Version: 1.00                            *

% * Date:   2017-04-29 00:34:32                     *

% * Coded By: Dan Bobczynski KG4HNS                   *

% *                                    *

% *************************************************************************

% *************************************************************************

% *************************************************************************

% Copyright (C) 2017 Dan Bobczynski KG4HNS

%

% This program is free software: you can redistribute it and/or modify

% it under the terms of the GNU General Public License as published by

% the Free Software Foundation, either version 3 of the License, or

% (at your option) any later version.

%

% This program is distributed in the hope that it will be useful,

% but WITHOUT ANY WARRANTY; without even the implied warranty of

% MERCHANTABILITY or FITNESS FOR A PARTICULAR PURPOSE. See the

% GNU General Public License for more details.

%

% You should have received a copy of the GNU General Public License

% along with this program. If not, see <http://www.gnu.org/licenses/>.

% 2017 Dan Bobczynski KG4HNS

% *************************************************************************

%

% The ReSearch.m program creates an expanded list of resistance

% values from a supplied list of resistors to aid in searching for parallel

% and series resistor combinations to form a desired resistance.

%

% It calculates the resistance of two resistors in parallel from a list of

% owned resistors. Each resistor is paired one at

% a time with the next resistors in the list. For each combination, a

% record is produced showing the computed parallel resistance, and the

% values of the two resistors used. This can then be used by a simple bin packing

% program to select and add up the desired resistance from the new resistance

% list. The bin packing program would then traverse this new list

% accumulating resistance records of value equal or less than the desired

% resistance until the desired resistance is approximated. The search

% depth determines the sample set size.

%

% The actual owned resistor list is also incorporated into the new list.

%

% The search depth determines the size of the output list, however the law

% of diminishing returns kicks in

% when going very deep, e.g, pairing a 1M resistor with a 1 Ohm resistor is

% of little value. The search depth limit should be determined by the

% percentage difference between paired resistances. A depth of six

% seems to accomodate most needs.

%

%

% Usage:

%

% The user must input; the name of the file containing

% the list of owned resistors; the search depth; the desired resistance;

% and the tolerance.

% The resistor values are then read into to the 'r' array.

% Upon completion, the new expanded resistance list resides in the

% 'nc' array. Each row of the 'nc' array

% contains: New resistance value; Resistor 1; Resistor2,

% where Resistor 1 and Resistor 2 are selected from the list of

% owned resistors. The nc array is then sorted, producing the

% ncsorted array. The desired resistance is then found by selecting and

% adding resistance values until the tolerance is met.

%

%

% run MakeResistors

clear                         % Clear the workspace

instring = input('Resistor list file name ? ','s'); % Input owned resistor file name

r = importdata(instring);               % Input owned resistance values

r = sortrows(r);                   % Sort the input resistance list

limitresolution = ('Resolution Limit Reached');    % Set up the resolution message

sdepth = input('Search depth ? (0 ends program) ');  % Input desired search depth

if sdepth == 0

exit                       % User inputs zero to quit

end

cdepth = sdepth;

dres = input('Desired resistance ? ');        % Input resistance desired

drestol = (input('Desired resistance tolerance in percent ? '))/100;

format shortEng                   % Set the output format

rsize=length(r);                   % Get the input array length

alloc = (rsize-sdepth)*(sdepth+2) + sum(2:sdepth+1);

n= zeros(1,alloc);      % Preallocation of new value list saves time

nc=zeros(alloc,3);      % Preallocate search results record list

ixn=1;                 % Initialize new list index

k=1;                  % Initialize resistor 2 index

for j=1:rsize             % Do for the number of owned resistors

if j > rsize - sdepth        % If not near the end, keep search depth the same

cdepth = cdepth - 1;      % If near the end, adjust search depth to equal remaining values

end

for i=0:cdepth           % Do for the deph of search

n(ixn)= 1/(1/r(j) + 1/r(i+k)); % Calculate the parallel resistance from resistor 1 and resistor 2

nc(ixn,1) = n(ixn);       % Fill out the new resistance record with computed value

nc(ixn,2) = r(j);        % Fill out the new resistance record with resistor 1

nc(ixn,3) = r(i+k);       % Fill out the new resistance record with resistor 2

ixn = ixn + 1;         % Increment the new list index

end                 % End of depth search

nc(ixn,1) = r(j);          % Insert the owned resistor value into the new list

ixn = ixn + 1;           % Increment the new list index

k= k+1;               % Increment the resistor 2 index

end                  % End of number of owned resistors

ncsorted= sortrows(nc);        % Sort the output resistance records on the result field.

% Results Format:

% Each row (record) of ncsorted contains the results of

% the parallel calculation and the values of the two owned

% resistors used in the calculation as follows:

%    Result value; Owned resistor #1; Owned resistor #2

%

% To obtain the resistance required, simply select resistors

% from the new list to be placed in series.

dresidx = find((ncsorted(:,1))<=dres); % find the record with largest resistance less than or equal to desired

dresix=dresidx(end);          % get the index to it

dressum=ncsorted(dresix,1);      % get the resistance from the sorted records

resseries(1) = ncsorted(dresix);    % initialize resseries (the resistances to use)

for m=1:ixn     % Now search records for values to place in series to make the desired resistance

residx = find(ncsorted(:,1) <= (dres - dressum));  % get index list of next value less than (desired - current sum)

if isempty(residx) | dressum == 0          % quit if sum is 0 or no next value

limitresolution

break

end

resix = residx(end);                % get last index from found list

dressum = ncsorted(resix) + dressum;        % add the new value to form desired resistance

resseries(m+1) = ncsorted(resix);         % append the found record to resistances to use

if abs(dres - dressum) <= drestol*dres       % if desired tolerance reached, quit

break

end

if (dres-dressum) <= ncsorted(1)          % quit if no smaller resistance can be found

no_smaller_resistor = ('TRUE')

break

end

end

format shortEng

Use_these_ncsorted_resistances = resseries'   % list out the series resistance values and composite resistors

To_obtain_this_resistance = dressum

%format bank

format shortG

Actual_Tolerance = ((dres - dressum)/dres)*100  % list the actual tolerance obtained

format shortE                  % set format for demo

% format shortEng                % set format for full output

ncsorted                     % list entire resistance list for demo

%end

% semilogy(ncsorted(:,1),'.')        % have a look at the resister

% hold on                  % pair differences vs th

% semilogy(r,'*')              % resulting resistance.

% hold off

% *******************************End Code******************************************